Cleanability:  Building construction and finish details are important to cleaning operations since specific designs and finishes lend themselves very well to a productive, efficient, well-managed and successful cleaning effort while others have the opposite impact.  As a result, the following comments are offered to promote “cleanability” within new and remodeled buildings.  The following types of surfaces often serve as dust catchers and are extremely hard to clean.

Rough - Textured Surfaces:  Exposed rough-textured surfaces, such as exposed brick, within buildings should be minimized.

Wood Trim:  Wood trim and chair rail shall be installed in moderation and only where needed.

Exposed Pipes and Raceways:  Exposed pipes, raceways, etc. (especially horizontal ones), as well as visible, but inaccessible, horizontal flat surfaces should be avoided.

Filler Panels:  Filler panels shall be provided at the ends of base cabinets and wall cabinets to close off narrow inaccessible spaces between them and adjacent walls.