By Department.

Offices:

| Director + 4-6 seat conference area, as required. | 165 nasf/FTE |
| --- | --- |
| Academic, Senior and tenured | 165 nasf/FTE |
| Associate Deans and Advisors | 150 nasf/FTE |
| Academic and non academic professional | 135 nasf/FTE |
| Senior Clerical | 80-120 nasf/FTE |

Open Landscape:  Space sizes are tailored for systems type furniture increments.  Support, part time, and hourly appointments:

| 25% appointment: | 6 x 8 = 48 sf |
| --- | --- |
| 50% appointment: | 8 x 8 = 64 sf |
| 75% appointment: | 8 x 10 = 80 sf |
| 100% appointment: | 10 x 10 = 100 sf |

Formula for calculating appointment %, then round to a, b, c, or d above:

Number hours appointment = % appointment x 120 sf = station size

40

Note:  All % appointments are summed to arrive at a total FTE.

(i.e.: .25 FTE + .75 FTE = 1 FTE)

FTE = Full Time Equivalent (40 hour/week employee)

Additional Space Allowances – for waiting, conference room, break and storage:

| For work room, conference, storage add: | 15 nasf/FTE |
| --- | --- |
| For receptionist, 0 – 5.99 FTE add: | 120 nasf |
| For conference room, 6 – 15.99 FTE add: | 200 nasf |
| For conference room additional, 16 – 26 FTE: | 50 nasf |
| ADD FACTOR, space size to be determined for special or unusual departmental requirements. Functions such as departmental computer or instructional (teaching) labs must be specifically programmed as to net assignable square feet (nasf) required for each individual function. |  |

Instructional Space – General Assignment Classrooms:

General Assignment Classrooms are not assigned to a specific department.  Departmental classrooms or labs are spaces not counted in the General Assignment classroom pool and are controlled by a specific Department.

| Classrooms with movable tablet arm chairs (MTAC) | Nominal size = 16 nasf/station |
| --- | --- |
| Classrooms and theaters with fixed seating | 10 nasf/station |
| Type, size, location and media requirements of General Assignment classrooms are a collaborative effort between the Classroom Scheduling Office and the respective Academic Departments. |  |
| Class Labs with strip tables and movable chairs | 24 – 28 nasf/station |

Note: Above spaces are net assignable square feet (nasf) only and do not include circulation, wall thicknesses, toilets, or building infrastructure.